Briarwood Avenue Drainage Upgrade
Authority Hierarchy and Public Lookup Instructions

Document BA-DGN-AUTH-003 | Revision A | Issued 23 May 2026

Use this file to resolve conflicts between the Briarwood Avenue project records. Public references verify methods and rainfall data only; they do not override project-specific survey, as-built, cost, legal, or programme constraints.

# 1. Source authority hierarchy

| Rank | Source class | File or record | Use |
| --- | --- | --- | --- |
| 1 | Signed mandatory constraints and owner decisions | Briarwood_Options_Costs_and_Constraints.xlsx | Controls budget ceiling, lifecycle ceiling, HGL freeboard, gutter spread, easement rule, closure limit and option-screening thresholds. |
| 2 | Approved survey datum correction and post-event invert survey | Briarwood_Survey_and_Invert_Register.xlsx | Controls corrected rim, invert and sump elevations after applying +0.42 m datum correction. |
| 3 | Revision D as-built drainage plans | Briarwood_Drainage_AsBuilt_Plans.pdf | Controls pipe sizes, lengths, inverts, manhole sequence and C-01 culvert geometry. |
| 4 | Storm event record and field photo log | Briarwood_Storm_Event_and_Field_Photos.docx | Controls field confirmation of C-06 exclusion, P-07 surface type, provisional tailwater and unresolved inspection gaps. |
| 5 | Vendor option quotes | Briarwood_Options_Costs_and_Constraints.xlsx | Controls option scope, capex, annual O&M, closure durations and quoted constraints. |
| 6 | Public-source lookup files | Referenced URLs in Section 4 | Use only for rainfall intensity lookup and published hydraulic method checks. |

# 2. Specific conflict and missing-information deductions

| ID | Issue | Conflict or missing fact | Required deduction |
| --- | --- | --- | --- |
| DAT-01 | Datum correction | SRV-FB-05 says +0.24 m, while SUR-DAT-2026-05 says +0.42 m. | Use +0.42 m because SUR-DAT-2026-05 in Briarwood_Survey_and_Invert_Register.xlsx is signed and supersedes the field-book note. |
| HYD-01 | Rainfall intensity | Draft hydrology notes use a 10-year event; the prompt requires a 25-year event. | Search NOAA PFDS for the project coordinates and use the 25-year 15-minute intensity. Do not reuse the 10-year draft value. |
| AREA-01 | Contributing drainage area | 2019 GIS map implies 3.20 ha; active catchment register totals 2.78 ha. | Use 2.78 ha because C-06 is diverted to Cedar Lane by 2025 curb works confirmed in Photo P-03. |
| AREA-02 | P-07 surface coefficient | Operations email describes P-07 as asphalt; field photo P-07 shows compacted gravel. | Use runoff coefficient 0.70 for C-05, not 0.90. |
| PIPE-01 | MH-2 to MH-3 pipe size | Legacy GIS asset table lists 450 mm; Rev D as-built plan lists 600 mm RCP. | Use the Rev D as-built plan value from Briarwood_Drainage_AsBuilt_Plans.pdf. |
| CULV-01 | C-01 culvert geometry | Field note says the outlet appears circular; Rev D detail shows 900 x 600 box culvert. | Use 900 x 600 box culvert from the sealed as-built detail and keep CCTV confirmation open. |
| TW-01 | Tailwater | Field mark gives 83.05 m but receiving-channel survey is incomplete. | Use 83.05 m as provisional 60% tailwater and record receiving-channel normal-depth verification as an open item. |
| OPT-01 | Option C easement | Draft owner email supports easement discussion; no signed easement is attached. | Do not treat Option C as available for recommendation unless a signed and recorded easement is provided. |
| OPT-02 | Option D budget and closure | Option D passes hydraulic values but exceeds capex and lane-closure limits. | Reject Option D for mandatory programme constraints even if hydraulic calculations pass. |

# 3. Required engineering calculations

| Area | Instruction |
| --- | --- |
| Hydrology | Calculate weighted runoff coefficient and Rational Method peak flow for the corrected active area. Use Q = 0.00278 C i A when i is in mm/h and A is in hectares. |
| Gutter spread | Use FHWA HEC-22 roadway drainage method for gutter spread. Check against REQ-SPREAD-01 maximum 1.80 m. |
| Storm drain capacity and HGL | Use corrected rim/invert elevations, Rev D pipe sizes, Manning n = 0.013 for RCP and 0.015 for older brick-lined catch basin leads unless a better project value is documented. |
| Culvert check | Use FHWA HDS-5 inlet-control and outlet/tailwater checks for C-01 with 900 x 600 box geometry. |
| Cost comparison | Use the Option_Quotes and Lifecycle_Costs sheets. Compute 30-year present value using the signed discount rate and annual O&M. |
| Recommendation | Screen Options A-D against every mandatory constraint. Recommend Option B only if it passes hydraulics, cost and ROW constraints and remaining open items are non-blocking for 60% design. |

# 4. Public-source references to search and cite

| File name | Summary | Parent website + document link |
| --- | --- | --- |
| NOAA Atlas 14 / PFDS precipitation-frequency estimates | Obtain the 25-year, 15-minute rainfall depth or intensity for the project coordinates. | NOAA/NWS Hydrometeorological Design Studies Center PFDS: https://hdsc.nws.noaa.gov/pfds/ |
| FHWA Hydraulic Design of Highway Culverts, HDS-5, Third Edition | Verify accepted culvert inlet-control, outlet-control, tailwater and headwater terminology and method selection. | FHWA HDS-5 PDF: https://www.fhwa.dot.gov/engineering/hydraulics/pubs/12026/hif12026.pdf |
| FHWA Urban Drainage Design Manual, HEC-22, Fourth Edition | Verify gutter spread, storm-drain/HGL and roadway drainage terminology and method selection. | FHWA HEC-22 PDF: https://www.fhwa.dot.gov/engineering/hydraulics/pubs/hif24006.pdf |

# 5. Deliverable requirements

| Output file | Required contents |
| --- | --- |
| Briarwood_Drainage_Reconciliation_Workbook.xlsx | Must contain source hierarchy, corrected inputs, hydrology, HGL, culvert, option screening, lifecycle cost, citations and open items. |
| Briarwood_Drainage_Recommendation_Memo.docx | Must state whether Option A, Option B, Option C or no option can be advanced to 60% design. |
| Briarwood_Drainage_Open_Items.csv | Must list unresolved facts, owners, decision gates, status and whether each item blocks 60% design. |
| Briarwood_HGL_Profile_and_Culvert_Check.pdf | Must show HGL from MH-2 through MH-5 and governing culvert headwater/tailwater check. |